Model 4 Disassembler System                                                                                  This program is a modified version of Mike Stilson's dissembler program,   along  with  a support program to aid in disassembly of CP/M-format ".COM" files  (COMHACK/BAS)  and  a  program  to change  the  internal  labels of assembly-language source files (ASMLABEL/BAS).                                                                                                                 DISASSEM/BAS                                                        Because of the modifications I have made to the program, it can no longer disassemble files from memory.                        The program runs from  BASIC.   After  it  starts,  a small title screen will be displayed along with several options.  The Symbol  Table  toggle  determines  whether  or not you want the disassembler to generate symbols for use by an editor/assembler (this  will require a two-pass disassembly).  The  disk  output toggle will  allow you to have the disassembler save its output to disk in one  of three formats (EDTASM, EDAS or MACRO-80).  I removed Mike Stilson's  M-ZAL  option,  since  in  the original program this would only cause a message to pop saying  "use the conversion  utility"  and  bounce  you  back  to  EDTASM format anyway.                                                             The block option is the most complicated.   This allows you to  set  up  to  thirty "blocks" of program code, specified  by their starting addresses  and assembly switch.  This allows you to tell the assembler where  a  block  of  DEFM's,  DEFB's,  or DEFW's should be generated instead of opcodes.  This is exactly the same as pressing the equivalent key during disassembly, but if you have already determined where the sections are (normally by  going  through  a  one-pass disassembly operation), you can have the computer disassemble  a  program  unattended  and feel reasonably confident it will do it correctly.  (Considering the program's  written  in BASIC, you'll probably want to let it do an unattended disassembly, as it may take several hours!)            During disassembly,  there will be a small menu at the top of the screen listing keys you  can  press  during operation to change  the output of the disassembler.  Switching  it  to  any generation  key  other  than opcodes will cause the Free/Single Step switch to be set to single-step.   (You  can  switch it to free  run  during  disassembly of any type, but it switches  to single  step  to  help prevent  missing  the  next  section  of opcodes.)                                                           The output file can have a maximum of 1800 lines.  When you go over that point, disassembly pauses and the program will ask for the name  of  another  output  file, and then continue with disassembly.                                                                                                                    COMHACK/BAS                                                         This rather odd  little  program  takes  a machine-language program stored in the CP/M ".COM" format and transfers  it into a  TRSDOS-readable  "/CMD" file.  Then, you can run the program through a disassembler and see what "makes it tick."  Note that even though it makes it a /CMD file, it's still a CP/M program, and  will  *not* run under  TRSDOS  without  modification:  the service calls  are  completely different, and all CP/M programs load at 0100H, which no  TRSDOS  6.2  program  does  unless  it doesn't  mind clobbering DOS.  (CP/M loads its operating system at the top of memory, and locations 0000-00FFH are pointers and system variables.   Keeping  a  standard I/O block like this is what allows CP/M to be ported onto  all different types of 8080 and Z-80 machines.)                                                 The only option in COMHACK/BAS regards deleting consecutive identical sectors.  In CP/M there is no  way  to  have one .COM file that has different modules loading in different  parts  of memory  (as  TRSDOS  allows,  with  one file having several ORG statements).  Programs that have separate modules actually save all  of the memory in between the modules,  too,  in  the  .COM file, and  this  "extra"  stuff  will confuse the hell out of a disassembler.  If two blocks of memory  are  exactly identical, COMHACK assumes only the first one is the "real"  block and the others are dummy ones to allow CP/M to put the modules  in  the correct  places.   Since  I'm  not at all sure this method will hold true for all CP/M files, this  is  an  option, rather than something done automatically.                                       If you don't have a CP/M manual this utility  will probably be  useless.   If  you  do, then the DISASSEM/BAS program  will automatically recognize an  input  file as being CP/M if it is, and will add a few relevant comment lines  when different parts of the low-memory I/O block are being referenced,  such  as the service call jump at 0005H.  Of course, since COMHACK generates standard /CMD files they can be run through any disassembler.         A  last  note  about CP/M files.  CP/M does not store the exact  end of the file (i.e.,  the  ending  byte  in  the  last sector),  so  it's  quite likely that the last few bytes of any file will be completely meaningless.                                                                                            ASMLABEL/BAS                                                        This program performs  a simple search-and-replace function on an assembly-language source  file, changing symbols from one name to another.  This is useful for changing the obscure names generated by disassemblers (or other  programmers) to ones that are more meaningful.                                                To use this program, use a text editor or the BUILD command to create a short ASCII text file with a  list of symbols.  The format  of  the  file  is:  "oldname","newname"<cr>.    For  an example,  see  the included CPM/LBL file, which can be used  to replace the symbols generated by DISASSEM/BAS for CP/M internal references with more standard names.                                ASMLABEL/BAS  also  includes  a simple way to include other label files inside the files read (the equivalent of a *INCLUDE directive).  This way, you could include  CPM/LBL (or perhaps a library of other standard names) in a label  name  file  that's specific   to   a   particular  source-code  file.   To  direct ASMLABEL/BAS to include  another  file,  have  a  line reading: *,filespec  <cr>    (where "filespec" is the full name  of  the file to include).   Included  files can have include directives into them, although the "nesting  level"  only  runs  up  to 15 (that's  the maximum number of files BASIC can have open at one time).